Electronic Rotary Table Divider.

Legal Stuff.

I offer all documents, code and schematics as an aid to the experienced constructor to allow them to build their own division controller. I offer no warranty either express or implied, i.e. you build and operate this device at your own risk. In building and operating this device you accept that there may be bugs or other problems both with the hardware, it’s associated software and documentation that may cause it to function in a manner that could possibly cause injury. None of the documentation, hardware design or software either in whole or part may be used for commercial purposes. Any re-distribution of this material must contain the original material in its original form, no charge may be made for its distribution.

All material is copyright Steve Ward 2006. Any questions or queries can be posted to me at steve@worldofward.com.

Description.

Device intended to drive a stepper motor controller using direction and step (clock) signals or via connection to a pair of H bridges. This allows the simple use of a rotary table without all the tedious mucking about with division plates.

Features.

- Jog mode. Keys move CW/CCW in 0.01, 0.1, 1.0, 10.0 degree steps.

- Division mode. Any number of divisions from 1 to 9999.

- Degree mode. Any number of degrees from 0.01 to 359.99.

- Continuous drive. Table is driven continuously CW/CCW.

- Configurable for any worm drive from 1:1 to 9999:1.

- Five speeds – min & max set by user, all others interpolated.

- CW direction swappable (for controller / motors that drive the ‘wrong way’).

- Clock direction swappable to suit controller.

- Five user profiles.

- Adjustable backlash compensation.

# Keypad

Keypad is 16 keys in a 4 x 4 arrangement as below.

| 1 -0.01 | 2 | 3 +0.01 | ###### Speed |
| --- | --- | --- | --- |
| 4 -0.1 | 5 | 6 +0.1 | ##### S |
| 7 -1.0 | 8 | 9 +1.0 | ###### G |
| << -10.0 | 0 | >> +10.0 | ###### C |

Throughout the following document, a keypress is shown in square brackets i.e. [1] means the key 1 etc.

[S] Is the stop key, if the table is moving pressing [S] will stop it immediately on the current step. Note: Inertia may mean the position can no longer be counted on to be accurate, so using the stop key invalidates the current position, division or angle.

[C] Is the cancel key, if the table is moving pressing [C] will decelerate it to a stop. Whilst editing numbers pressing [C] will cancel the current edit and restore the original value. If in a menu or mode, pressing [C] will generally take you up a level.

[G] Is the enter key. In the division and degree modes pressing [G] will allow you to edit the division or angle show thereby allowing you to go directly to a division or angle. In setup pressing [G] will allow you to change the displayed parameters. When editing a number pressing [G] will enter the currently shown number.

[Speed] In modes where the speed is shown (top right of display) pressing speed will cycle through the numbers 1 to 5. Where 1 is slow and 5 is fast (as set in setup), 2,3 & 4 are interpolated values based on the figures entered for 1 & 5.

The values under the numeric keys 1,4,7,3,6 & 9 are used in jog mode and pressing one of these keys in that mode moves the table by the amount shown (degrees).

Editing numbers. When a number is selected to be edited it will be surrounded by two square brackets (i.e. [0060]), a curser will also be visible beneath one of the numbers (starting at the left). The curser can be moved left and right using [<<] & [>>] and pressing a number key will overwrite the value displayed at that position. (When a number includes a decimal point it will not be possible to change the position of the point or overwrite it with a numeric value).

Pressing [C] whilst editing will cancel the current edit and the number will be restored to its original value, pressing [G] will enter the current number and complete the edit.

Getting started.

To familiarise yourself with the software, hook up the supply lines, switch on and admire the splash screen, after a couple of seconds the display should prompt you for a profile – either enter a number of press enter [G] to select the default, the menu should now look like the one below..

Once you’re happy with the software you can now hook up your controller to the clock and direction lines – alternatively hook up a pair of H-bridges to the 4 H-bridge lines.

1>Jog		9> Setup

2> Division

3> Degrees

4> Continuous

To test correct operation go into the Jog screen (press [1]) and press the ‘9’ key (+1.0 degrees). Display will briefly flash “Working”.

Ideally the table should turn CW through 1 degree exactly, if it does then congratulations – read the rest of the manual and play, otherwise: -

If the table doesn’t turn at all.

Check controller and stepper motor is ok, i.e. powered up. If that fails change the polarity of the step pulse in setup (remember to save the new settings).

Table turns but CCW.

Go into setup and swap the direction of the table.

Table turns, but not by one degree.

Go into setup and change the worm ratio to match yours (default worm is 90:1).

Note: stepper is assumed to be 400 steps per turn (i.e. 200 step stepper in half step mode), if this isn’t true you must calculate a worm ratio to fit. I.e. 72:1 worm with 800 steps per rotation stepper is the same as a 144:1 worm with a 400 steps per rotation stepper.

Table turns, but not smoothly and possibly noisily.

Step rate is higher than your motor will allow and/or table is tight or obstructed. If the first change the step rates in setup.

Jog Mode.

In jog mode the following screen is displayed.

#### Jog:		Spd: 1

Pos:  0.000

Spd shows currently selected speed; pressing the [Speed] key runs through available speeds  (1 – 5). Speed 1 represents the values entered for ‘Min Speed’ in setup, whilst 5 represents the values entered for ‘Max Speed’ in setup, all other values are interpolated from these two.

Position shows the current table position in degrees. When coming into this mode this value isn’t zeroed, so will hold the last value from a previous mode.

In this mode the user can press the following keys.

Keys [1] & [3] to move CCW / CW by 1/100th of a degree.

Keys [4] & [6] move through 1/10th degree.

Keys [7] & [9] to move through 1 degree.

Keys [<<] & [>>] to move through 10 degrees.

Whilst the table is moving press either [C] or [Stop] to stop the table. ([C] Decelerates to stop whilst [Stop] halts the table at the current step).

To exit jog mode, press [C] (cancel).

Division Mode.

When entering division mode the user is prompted for the number of divisions (per 360 degrees). The default value will be the last value set (or 60 if first time in). Edit this number to show the required number of divisions and press [G].

You will now get the following screen.

Division:	Spd: 5

#### Pos: 0.000

Div:	0 of 60

Note: On entering division mode, position is always reset to 0.000. Current division will be zeroed and number of divisions will represent value entered in previous screen.

Speed shows currently selected speed (value will be last value used or default of 5), this can be changed by repeatedly pressing the [Speed] key.

Pressing either the [<<] (CCW) or [>>] (CW) keys will advance the table one division with the display updating to reflect this. The display will show ‘Working’ whilst the table is moving.

Pressing the [G]o key will allow you to edit the current division number (thereby allowing you to skip direction to a given division), press either [G]o or [C]ancel.

You will now see a prompt ‘-Press Direction-‘, press either the [<<], [>>], [C] or [G] to move CCW, CW, cancel or take the ‘fastest route’ (i.e. either CCW or CW depending on which direction is fastest) respectively.

Whilst the table is moving press either C or Stop to stop the table. (C decelerates to stop whilst ‘Stop’ halts the table at the current step).

# Degrees Mode

When entering degrees mode the user is prompted for the number of degrees to be used for each division. Enter a number between 0.01 & 359.99 and press the [G] key.

The screen now looks something like this:

### Degrees:	Spd: 5

Pos: 0.000

Deg:  60.00

Note: On entering degree mode, position is always reset to 0.000. Number of degrees will represent the value entered in the previous screen.

Speed shows currently selected speed (value will be last value used or default of 5), this can be changed by repeatedly pressing the ‘Speed’ key.

Pressing either the << (CCW) or >> (CW) keys will subtract or add Deg from Pos and move the table to that position. The display will show ‘Working whilst moving’.

Pressing the [G]o key will allow the editing of the Deg field to allow you to move directly to the angle entered (it does not change the original value entered for degrees, this will revert after the operation has finished).

Once you’ve entered a new angle, press either [C] to cancel or [G] to go to that angle.

You will now see a prompt ‘-Press Direction-‘, press either the [<<], [>>], [C] or [G] to move CCW, CW, cancel or take the ‘fastest route’ (i.e. either CCW or CW depending on which direction is fastest) respectively.

Whilst the table is moving press either C or Stop to stop the table. (C decelerates to stop whilst ‘Stop’ halts the table at the current step).

# Continuous Mode

When entering continuous mode the display changes to:

#### Continuous:	Spd: 4

Press Direction

Speed shows currently selected speed (value will be last value used or default of 4), this can be changed by repeatedly pressing the [Speed’ key].

Press either [<<] or [>>] (CCW / CW) to start the table moving in the appropriate direction.

Whilst the table is moving press either [C] or [Stop] to stop the table. ([C] Decelerates to stop whilst [Stop] halts the table at the current step).

# Setup

On entering setup the following options are available – press either [<<] or [>>] to move forward or backward through the list. Press [C] to exit back to the main menu.

Set Min Speed.

Set Max Speed.

Set Worm Ratio.

Swap Rotation Dir.

Set Backlash.

Swap Clock Pulse.

Change Profile.

Save and Exit.

Factory Defaults.

## Factory Defaults

This restores all settings to the internal defaults. Press [G] to execute this.

## Set Min Speed

Values used for min speed setting (1). Shows current settings for Lo, Hi and Ramp. (Stepper starts at Lo steps per minute and each step adds Ramp until it reaches Hi). Press [G] to edit these values. (Hi is constrained to be no lower than Lo).

Note:

Too large a value for Lo will make it difficult for the stepper to begin rotation from standstill resulting in lost steps, too low will take longer to reach speed.

Too high a value for Hi will cause the stepper to misstep, this will cause the table to jerk and steps to be lost, too low and the table may not spin at the desired speed.

Too high a value for Ramp may make it difficult for the stepper to reach the higher speed causing lost steps. Too low and the table may take to long to reach the high speed.

## Set Max Speed

Values used for max speed setting (5). Shows current settings for Lo, Hi and Ramp. (Stepper starts at Lo steps per minute and each step adds Ramp until it reaches Hi). Press [G] to edit these values. (Hi is constrained to be no lower than Lo).

Notes for min speed also apply here.

## Set Worm Ratio

Shows current worm ratio.

Press [G] to edit this.

Note: stepper is assumed to be 400 steps per turn (i.e. 200 step stepper in half step mode), if this isn’t true you must calculate a worm ratio to fit. I.e. 72:1 worm with 800 steps per rotation stepper is the same as a 144:1 worm with a 400 steps per rotation stepper.

## Swap Rotation Dir

Allows the controller to switch what it thinks is the CW & CCW directions, useful if your controller & hardware rotate the wrong way…

Press [G] to swap between the two.

## Set Backlash

Shows current backlash settings. Press [G] to edit this.

Backlash is applied to CCW rotations; it should be a number slightly bigger than the number of steps needed to take up the backlash. When rotating CCW the controller will add this number to the number of steps required then move CW by the same number – hence removing the backlash by always finishing in a CW direction, much the same as with manual operation.

Note: Too high a value for backlash results in extra unnecessary movement, too low and the backlash isn’t taken up resulting in inaccuracy.

## Swap Clock Pulse

This allows some customisation for different controllers. Some controllers expect the clock line to be held low momentarily pulsing high to clock the stepper whilst others work the other way around. The default is ‘High’, pressing [G] toggles between ‘High’ and ‘Low’.

Note: The wrong value here may prevent the controller from stepping correctly (or at all). Some controllers will work ok regardless of the polarity of the clock pulse.

## Change Profile

Allow you to change between one of the five user profiles. These are stored in EEPROM on the CPU and thus maintain the settings when power is removed. Normal use would be to use a different profile on each device you connected the controller to. I.e. rotary table, lathe head indexer etc.

Note: Remember to save your current settings if they’ve changed before switching profiles, otherwise any changes you’ve made will be lost.

## Save and Exit

Pressing [G] will save any values changed into the current profile. This includes any speed values that have been changed in the various modes, default values entered for degree and division modes and all settings under the setup menus.